# Pokhara University

| Level: Bachelor | Semester: Fall | Year : 2016 |
| --- | --- | --- |
| Programme: BCA |  | Full Marks: 100 |
| Course: Digital Logic System |  | Pass Marks: 45 |
|  |  | Time: 3hrs. |

Candidates are required to give their answers in their own words as far as practicable.

The figures in the margin indicate full marks.

## Attempt all the questions.

|  | - Perform the following conversions. - (101)10 = (?)2 - (BAF)16 = (?)8 - (100110101)gray = (?)2 - (142)10 = (?)BCD - (4565)8 = (?)16 - Show the universality of NOR gate. | 10 5 |
| --- | --- | --- |
|  | - State and prove the three bit De-Morgan’s theorem - Design a combinational logic circuit that accepts a three-bit number and generates an output binary number equal to the square of the input number. | 7 8 |
|  | - Design a combinational logic circuit to convert BCD code to excess-3 code. - Implement a full adder using 3×8 decoder and OR gates. | 7 8 |
|  | - What is Rs flip-flop? Explain its operation with the help of logic circuit and truth table. 00 01 10 11 1 0 0 1 0 0 1 1 Design sequential circuit using JK flipflop. | 8 7 |
|  | - Design 3 bit up/down synchronous counter using T flipflop. - Explain the operation of serial in parallel out (SIPO) shift register with the help of necessary circuit diagram. - Differentiate between synchronous and asynchronous counter. | 5 5 5 |
|  | - Explain in brief about ALU. - Design a 4-bits adder/sub-tractor logic circuit. - What is memory stack? Explain PUSH and POP operation. | 5 5 5 |
|  | Write short note on any two: - Ripple counter - Alphanumeric Code - Significance of K-map | 2×5 |