# Pokhara University

| Level: Bachelor | Semester – Fall | Year : 2013 |
| --- | --- | --- |
| Programme: BCA |  | Full Marks: 100 |
| Course: Digital Logic Systems |  | Pass Marks: 45 |
|  |  | Time : 3hrs. |

Candidates are required to give their answers in their own words as far as practicable.

The figures in the margin indicate full marks.

## Attempt all the questions.

|  | - Perform the following conversions: - (24.6)10=(....)2 - (0.582)H=(....)10 - (3B7-854)H=(....)8 - (01001001011)gray=(....)2 - What do you mean by analog and digital system? Which system do you prefer and why? | 2×4 3+4 |
| --- | --- | --- |
|  | - State and prove De'Morgan's theorem. - Simplify the following Boollean expression to a minimum number of literals and implement with basic gates: ABC+A'B+ABC' - Find the SOP for the given truth table: X Y Z; F. 0 0 0; 0. 0 0 1; 1. 0 1 0; 1. 0 1 1; 1. 1 0 0; 0. | 5 5 5 |
|  | - Using K-Map method simplify the following Boolean function and implement using NAND gates only: F=A'B'D'+A'CD+A'BC d=A'BC'D+ACD+AB'D' - Design a combinational circuit that has four inputs and one output. The output is high only when the majority of inputs are high. OR What is multiplexer? Implement the following Boolean function using 8×1MUX:F=m0,1,5,7,12,14. | 8 7 7 |
|  | - Design a full adder using two half adder and an OR-gate. - Explain the advantage of JK Flip Flop over RS Flip Flop. Explain the working of Master Slave JK flip flop with diagram. OR Design a sequential circuit corresponding to the given state diagram using J-K flipflop. | 7 8 8 |
|  | - What do you mean by triggering of flip flop? Compare and contrast state table and state diagram. - What do you mean by shift registers? Explain the operation of PISO shift register with suitable diagram. | 2+5 8 |
|  | - Design a 4-bit ALU and explain its operation. - Design a 4-bit synchronous up down counter using T-flip flops. | 7 8 |
|  | Write short notes any two: - Processor Bus Organization - Number System - Steps to be followed while designing a combinational logic | 2×5 |